Conner Peripherals, Inc.

CP3000 Intelligent Disk Drive

Product Manual

Revision II

October, 1990

3081 Zanker Road

San Jose, CA   95134-2128

(408) 456-4500

Notice

Conner Peripherals makes no warranty of any kind

with regard to this material, including, but not

limited to, the implied warranties of merchantability

and fitness for a particular purpose.

Conner Peripherals shall not be liable for errors contained

herein or for incidental consequential damages in connection

with the furnishing, performance, or use of this material.

Conner Peripherals, Inc. reserves the right to change,

without notification, the specifications contained in this manual.

Copyright Conner Peripherals, Inc. No part of this publication

may be reproduced or translated into any language in any form

without the written permission of Conner Peripherals, Inc.

IBM, PC/AT and PC/XT are registered trademarks of

International Business Machines Corporation.

Table of Contents

1.0      Scope of Manual

2.0      Key Features

3.0      Specification Summary

3.1      Capacity

3.2      Physical Configuration

3.3      Performance

3.4      Read/Write

3.5      Power Requirements (Typical)

3.6      Physical Characteristics

4.0      Environmental Characteristics

4.1      Temperature

4.2      Humidity

4.3      Altitude (relative to sea level)

4.4      Reliability and Maintenance

4.5      Shock and Vibration

4.6      Magnetic Field

4.7      Acoustic Noise

4.8      Safety Standards

5.0      Functional Description

5.1      Read/Write and Control Electronics

5.2      Drive Mechanism

5.3      Air Filtration System

5.4      Head Positioning Mechanism

5.5      Reas/Write Heads and Disks

5.6      Error Correction

5.7      Customer Options

6.0      Electrical Description

6.1      Power Connectors

6.2      Power Connector Pin Description

6.3      Task File Interface Connector

6.4      Diagnostic Routines

7.0      Recommended Mounting Configuration

7.1      Shock Tolerance Features

7.2      Mechanically Isolated Mounting Points

8.0      Electrical Description

8.1      Signal Levels

8.2      Signal Conventions

8.3      Pin Descriptions

1.0      Scope of Manual

This manual describes the key features, specification

summary, physical characteristics, environmental

characteristics, functional description, electrical

interface, recommended mounting configuration, timing

requirements, Host address decoding, command description,

operations description, and error reporting for the

Conner Peripherals model CP3000.

2.0      Key Features

The CP3000 is a high performance 3.5 inch low-profile (1.00")

42.8 megabyte (formatted) disk drive with a 28 ms average seek

time. It is designed to operate on an IBM PC/AT or equivalent

in translate mode.

Key features of the CP3000 include:

o        Low power requirements enabling battery

operation in portable environments

o        High performance rotary voice coil actuator with

embedded servo system

o        High shock resistance

o        Internal air filtration system

o        Sealed HDA

o        Automatic actuator latch against inner stop upon

power down

o        Microprocessor-controlled diagnostic routines that

are automatically executed at start-up

o        Automatic error correction and retries

o        Block size:  512 bytes

o        Emulates IBM Task File and supports additional

commands

o        Up to two drives may be daisy-chained on this

interface

o        Translate mode (17 sectors, 5 heads, 980 cylinders)

is supported

o        1:1 interleave

o        Look Ahead Read Capability

o        AC Hysteresis on interface

o        8K Buffer

3.0      Specification Summary

3.1  Capacity

42.8 Mbytes       Formatted

3.2  Physical Configuration

Actuator Type                       Rotary Voice-Coil

Number of Disks                     1

Data Surfaces                       2

Data Heads                          2

Servo                               Embedded

Tracks per surface                  1045

Track Density (TPI)                 1400

Bytes per block                     512

Sectors per Track          40

The physical parameters of the drive are 1045 cylinders,

2 heads, and 40 sectors. At power-up, the drive will

default to the native mode for this drive, which is

522 cylinders, 4 heads, and 40 sectors.

This drive supports a translate mode of 980 cylinders,

5 heads, and 17 sectors. Drives with microcode level 2.32

and above will also support a translation emulating the

CP344 native parameters of 805 cylinders, 4 heads, and

26 sectors. (refer to Initialize Drive Parameters command).

3.3  Performance

Seek Times        Trk. to Trk.      11.0ms

Average           28.0ms

Maximum           50.0ms

Rotational Speed           3557 RPM

Data Trans. Rate           1.5 Mbytes/sec

Start Time                 Typical  5 sec

Max.     10 sec

Stop Time                  Typical 5 sec

Max      10 sec

Interleave                 1:1

Buffer Size                8K

The timing is measured through the interface with the drive

operating at nominal DC input voltages. The timing also

makes the following assumptions:

o BIOS overhead and PC system hardware dependency have

been subtracted from timing measurements.

o The drive is operated using its native drive parameters.

The average seek time is determined by averaging the seek

time for a minimum of 1000 seeks of random length over the

surface of the disk.

These numbers assume spin recovery is not invoked. If

spin recovery is invoked, the maximum could be 40 seconds.

Briefly removing power can lead to spin recovery being            invoked.

3.4  Read/Write

Interface                  Task File

Recording Method  2 of 7 RLL code

Recording Density 30,871 bpi

Flux Density               20,581 rpi

3.5  Power Requirements (Typical)

+12VDC   +5VDC    Power

R/W Mode 230ma    275ma    4.2 W

Seek Mode         140ma    180ma    2.8 W

Idle Mode         120ma    120ma    2.0 W

Standby Mode      10ma     90ma     0.5 W

Spin-up Mode      700ma    180ma    N/A

Read/Write Mode:  Occurs when data is being read from or

written to the disk.

Seek Mode:  Occurs while the actuator is in motion.

Idle Mode:  Occurs when the drive is not reading, writing

or seeking. The motor is up to speed and DRIVE READY condition

exists. Actuator is residing on last accessed track.

Standby Mode:  Occurs when the motor is stopped, actuator

parked and all electronics except interface control is in

sleep state.  STANDBY MODE will occur after a programmable

time-out since last Host access occurs.  Drive ready and seek

complete status exist.  The drive will leave STANDBY MODE upon

receipt of a command which requires disk access or upon receipt

of a spin up command.

Spin-up Mode: Occurs while the spindle motor is accelerating

from its state to its operational speed. During the typical

spin-up cycle, current on the 12 volt line may reach up to 900 mA

for up to 500 microseconds. The specified current is the averaged

value over the spin-up cycle.

Maximum noise allowed (DC to 1 MHZ, with equivalent resistive load)

3.6  Physical Characteristics

Dimensions        1.00" x 4.00" x 5.75"

Weight            1.1 Pound

4.0      Environmental Characteristics

4.1  Temperature

Operating                  5 deg. C to 55 deg. C

Non-operating              -40 deg. C to 60 deg. C

Thermal Gradient  20 deg. C per hr. (Max)

4.2  Humidity

Operating                  8% to 80% Noncondensing

Non-operating              8% to 80 Noncondensing

Max. Wet Bulb              26 deg. C

4.3  Altitude (relative to sea level)

Operating                  -200 to 10,000 feet

Non-operating (max)        40,000 feet

4.4  Reliability and Maintenance

MTBF                       100,000 hrs. (POH)

MTTR                       10 Min. typical

Maint.                     None

4.5  Shock

Non-operating              75 Gs

Operating                  5 Gs*

*Without nonrecoverable errors

4.6  Magnetic Field

The externally induced magnetic flux density may not

exceed six gauss as measured at the disk.

4.7  Acoustic Noise

40 dBA at a distance of 1 meter from the drive.

4.8      Safety Standards

The drive is designed to comply with relevant product

safety standards such as:

o  UL 478, 5th edition, Standard for Safety of Information

Processing and Business Equipment, and UL 1950, Standard

for Safety of Information Technology Equipment

o        CSA 22.2 #154, Data Processing Equipment and

CSA 22.2 #220, Information Processing and Business Equipment.

o        IEC 435 Safety Requirements for Data Processing Equipment,

IEC 380, Safety of Electrically Energized Office Machines, and

IEC 950, Safety of Information Technology Equipment Including

Electrical Business Equipment.

o        VDE 0805 Equivalent to IEC 435,

VDE 0805 TIEL 100, Equivalent to IEC 950, and

VDE 0806, Equivalent to IEC 380.

5.0      Functional Description

The CP3000 contains all mechanical and electronic parts

necessary to interpret control signals, position the recording

heads over the desired track, read and write data, and provide

a contaminant free environment for the heads and disks.

5.1  Read/Write and Control Electronics

One integrated circuit is mounted within the sealed enclosure

in close proximity to the read/write heads.  Its function is

to provide one of two head selections, read pre-amplification,

and write data circuitry.

The single microprocessor controlled circuit card provides the

remaining electronic functions which include:

o        Read/Write Circuitry

o        Rotary Actuator Control

o        Interface Control

o        Spin Speed Control

o        Dynamic Braking

o        Power Management

At power down or the start of STANDBY MODE, the heads are

automatically retracted to the inner diameter of the disk.

There they are latched and parked on a landing zone that is

inside the data tracks.

5.2  Drive Mechanism

A brushless DC direct drive motor rotates the spindle at

3557 rpm.  The motor/spindle assembly is balanced to provide

minimal mechanical runout to the disks and to reduce vibration

of the HDA.  A dynamic brake is used to provide a fast stop to

the spindle motor when power is removed, or upon initiation of

STANDBY MODE.

5.3  Air Filtration System

The head-disk assembly is a sealed enclosure with an integral

0.3 micron filter which maintains a clean environment for the

heads and disks.

5.4  Head Positioning Mechanism

The two read/write heads are supported by a mechanism coupled

to the voice coil actuator.

5.5  Read/Write Heads and Disks

Data is recorded on one 95 mm diameter disk through two 3370

type composite ferrite heads.

5.6  Error Correction

The CP3000 performs internal error correction.  The error

correction polynomial is capable of correcting one error burst

with a maximum of 8 bits per 512 byte block.  The following

polynomial is used:

Forward:  P(X) = (X32+X28+X26+X19+X17+X10+X6+X2+1)

5.7  Customer Options

There are four jumper options available for configuration:

HSP, C/D, DSP, and ACT.

HSP, when jumpered, connects the -HOST SLV/ACT signal on

the interface to ground for systems that require the slave

drive to provide -SLAVE PRESENT signal in a two-drive system.

C/D is the address jumper.  When jumpered, the master

(or C drive), is selected. When open, the slave (or D drive),

is selected.

DSP, when jumpered, indicates to the master drive that a slave

is present.  In a two-drive system, this jumper option must be

installed in the master, or C drive.

The last jumper, ACT, connects the -ACTIVE signal to the

-HOST SLV/ACT signal on the interface.  This signal provides

the capability to drive an external LED.  An external current

limiting resistor is required.

As an alternative, an LED may be connected between J4, pins 1

and 2.  This would provide both an open collector drive signal

and a current limiting resistor connected to +5V respectively.

Note 1:  In a two drive system, it is possible to drive one LED

with both drives.  An external current limiting resistor

is required.

Note 2:  If the drive is connected to a Host that requires that

the signal  -DRIVE SLAVE PRESENT be supplied from the slave

drive, via the interface signal -HOST SLV/ACT, then this

jumper must be installed.  If this jumper is installed,

the ACT jumper must not be installed because the two jumpers

are mutually exclusive.

6.0      Electrical Interface

6.1  Power Connectors

The CP3000 has a 4-pin DC power connector (J3) mounted on the

PCB. The recommended mating connector is AMP part number 1-480424-0

utilizing AMP pins (part number 350078-4 or equivalent).

DC power may also be supplied to the drive through a 3 pin

connector (J5). The recommended mating connector is Molex part

number 39-01-0033 utilizing Molex pins (part number 39-00-0031

or equivalent).

Power is to be supplied at either J3 or J5, but not both.

6.3  Task File Interface Connector

The CP3000 has a 40-pin Task File Interface connector (J2)

mounted on the printed circuit board.  The recommended mating

connector is Molex part number 10-91-2401 or equivalent.

Two drives may be daisy chained together at this connector.

Maximum cable length is two feet.

6.4  Diagnostic Routines

The microprocessor performs diagnostics upon application of

power. If an error is detected, the CP3000 will not come ready.

7.0 Recommended Mounting Configuration

The CP3000 drive is designed to be used in applications where

the unit may experience shock and vibrations at greater levels

than larger and heavier disk drives.

7.1  Shock Tolerance Features

The design features which allow greater shock tolerance are

the use of rugged heads and media, a dedicated landing zone,

closed loop servo positioning, and specially designed motor

and actuator assemblies.

7.2  Mechanically Isolated Mounting Points

Ten base mounting points are provided to the customer.

The drive is mounted using 6-32 screws; 1/8" maximum insertion

for the sides, and 1/4" maximum insertion for the bottom.

The system integrator should allow ventilation to the drive to

ensure reliable drive operation over the operating temperature

range.  The drive may be mounted in any attitude.

For additional vibration isolation, an external suspension

system may be used.

8.0      Electrical Description

8.1  Signal Levels

All signal levels are TTL compatible.  A logic "1" is greater

than 2.0 volts.  A logic "0" is from 0.00 volts to .70 volts.

8.2  Signal Conventions

The interface between the drive adapter and the drive is called

the Host Interface. The set of registers in the I/O space of the

Host is known as the Task File.

All signals on the Host Interface shall have the prefix HOST.

All negatively active signals shall be further prefixed with

a "-" designation.  All positive active signals shall be prefixed

with a "+" designation.  Signals whose source are the Host, are

said to be "outbound" and those whose source is the drive, are

said to be "inbound".

8.3  Pin Descriptions

The following table describes all of the pins on the Task File

Interface:

Signal Name       Dir      Pin      Description

- HOST RESET      O        01       Reset signal from the Host system which

is active low during power up and

inactive thereafter.

GND               O        02       Ground between drive and Host.

+HOST DATA        I/O      03-18    16-bit bi-directional data bus 0-

16 between the Host and the drive.

The lower 8 bits, HD0 - HD7, are

used for register and ECC access.

All 16 bits are used for data

transfers.  These are tri-state

lines with 24 mA drive capability.

GND               O        19       Ground between drive and Host.

KEY               N/C      20       An unused pin clipped on the drive

and plugged on the cable. Used to

guarantee correct orientation of the

cable.

RESERVED O        21

GND               O        22       Ground between drive and Host.

- HOST IOW        O        23       Write strobe, the rising edge of

which clocks data from the Host

data bus, HD0 through HD15, into

a register or the data register

of the drive.

GND               O        24       Ground between drive and Host.

- HOST IOR        O        25       Read strobe which, when low, enables

data from a register or the data

register of the drive onto the Host

data bus HD0 through HD15.  The

rising edge of - HOST IOR latches

data from the drive at the Host.

GND               O        26       Ground between drive and Host.

RESERVED O        27,29

+HOST ALE         O        28       Host Address Latch Enable. A signal

used to qualify the address lines.

This signal is presently not used by

the drive.

GND               O        30       Ground between drive and Host.

+HOST IRQ14       I        31       Interrupt to the Host system,

enabled only when the drive is selected,

and the Host activates the - IEN bit in

the Digital Output register. When the

-IEN bit is inactive, or the drive is

not selected, this output is in a high

impedance state, regardless of the state

of the IRQ bit.  The interrupt is set

when the IRQ bit is set by the drive

CPU. IRQ is reset to zero by a Host

read of the Status register or a write

to the command register.  This signal

is a tri-state line with 8 mA drive

capacity.

- HOST IO16       I        32       Indication to the Host system that

the 16 bit data register has been

addressed and that the drive is

prepared to send or receive a 16

bit data word. This line is a

tri-state line with 24 mA drive

capacity.

- HOST PDIAG      I        34       Passed diagnostic. Output by the drive

if it is strapped in the slave mode

(C/D not jumpered). Input to the drive

if it is strapped in the master mode

(C/D jumpered).  This low true signal

indicates to a master that the slave

has passed its internal diagnostic

command. This line is a tri-state line

with 24 mA drive capability.

+HOST A0,         O        35,      Bit binary coded address used to select

A1, A2                33,36    the individual registers in the task

file.

- HOST CS0        O        37       Chip select decoded from the Host address

bus.  Used to select some of

the Host accessible registers.

- HOST CS1        O        38       Chip select decoded from the Host

address bus. Used to select three of

the registers in the Task File.

- HOST SLV/ACT    I        39       Signal from the drive used either to

drive an active LED whenever the disk

is being accessed or as an indication

of a second drive present.  (See the

Customer Options section for further

information.)  When jumpered as

-ACTIVE, this signal is active low

when the drive is busy and has a drive

capability of 20 mA.  When jumpered as

-SLAVE PRESENT                      signal, it is an indication of the

presence of a second drive when low.

In this state, it has a

drive capability of 10 mA open drain.

GND               O        40       Ground between drive and Host.