COPYRIGHT (c) 1994 WESTERN DIGITAL CORPORATION

WDCDRV.DOC

READ ME FILE FOR WDCDRV.386

WDCDRV.386

Version:    2.4

Date:       6/8/94

Size:       16,475 BYTES

DESCRIPTION

WDCDRV.386 is a "Fastdisk" hard disk driver for use under Microsoft's

Windows 3.1 and Microsoft's Windows For Workgroups 3.11 on 386 or higher

based systems. WDCDRV.386 is a direct replacement for the built-in

Microsoft 32-Bit Disk Access driver WDCTRL.

Its main benefit is its ability to work with drives with more than 1,024

cylinders and capacities greater than 528 Million bytes (516 Megabytes).

WDCDRV.386 also has the ability to transfer data from the hard disk in either

a Block Mode PIO fashion, EISA type 'B' DMA fashion, or PCI type 'F' fashion.

Block Mode PIO is a means of transferring multiple sectors at a time between

the host and hard disk as opposed to transferring one sector at a time.

Normally in an AT compatible machine a hardware interrupt occurs for each

sector transferred to/from the hard disk. Under Block Mode PIO one interrupt

will occur for a block of sectors. The number of sectors that are to be

transferred per interrupt is programmable by the host and is referred to as

the blocking factor. The driver will determine if block mode is enabled for

the drive when you start Windows. If block mode is detected then the blocking

factor programmed is used as the default value for the Fastdisk driver. If

block mode was not enabled for the drive when Windows was started the default

blocking factor used equals the maximum blocking factor supported that is

reported by the drive on an Identify Drive command.

Transferring data via Block Mode PIO increases throughput by eliminating

the inter-sector overhead associated with each sector. This is most

beneficial when the hard disk is transferring data out of its cache.

Block Mode data transfers are enabled by default.

This driver also supports EISA type 'B' DMA transfers for drives that

support this feature and are installed in an EISA system. Data is

transferred at a rate of 4 MB/sec which is about double that of a

6 MHz AT. The standard Read DMA and Write DMA commands as described in

the CAM ATA specification are used.

Furthermore, type 'F' DMA is supported in PCI based systems. Data is

transferred at a rate of between 6 and 8 MB/sec using type 'F' DMA.

The same Read DMA and Write DMA op codes are used, the only difference

is in the programming of the DMA controller on the motherboard.

Transferring drive data via DMA allows the host CPU to work on other tasks

while hard disk data transfers are taking place. This increases the

overall CPU bandwidth which is critical in multi-tasking environments.

This driver only runs Block Mode PIO and DMA on Western Digital drives.

To use the DMA feature the drive must be installed in an EISA or PCI

system and the DMARQ and DMACK- signals on the the 40 pin cable must be

connected to the proper DMA request and acknowledge signals on the EISA

or ISA bus.

This revision of the WDCDRV driver automatically supports Logical Block

Addressing (LBA) on drives that support this feature. When Translating BIOS

is detected in the system, the driver will support the drive in LBA mode if

LBA is supported by the drive, otherwise it will support the drive in

Translating CHS mode.

This revision of WDCDRV drive automatically detects DMA channel and system

bus type being EISA, PCI/ISA or PCI/EISA.

NOTES ON OPERATION

There are several issues related to operation of the driver that the user

may need to be aware of. These are detailed in this section and may cover

potential incompatibilities or may simply be more detailed information on

driver operation or installation.  Not all information will be needed by

every user and may often be skipped or ignored.

Known Problems:

1) This driver will not operate with the version of EMM386.EXE supplied with

MS-DOS 5.0.  This version should not be in use since Windows 3.1 setup

will replace the lines in CONFIG.SYS to load HIMEM.SYS and EMM386.EXE

with lines to load these files from the Windows directory.  New versions

of these files supplied with Windows 3.1 operate correctly with WDCDRV.386.

The versions of EMM386.EXE supplied with MS-DOS 6.0 and MS-DOS 6.2 have

also been verified to operate correctly with WDCDRV.386.

Operational Notes:

1) The driver will always operate in LBA mode if the drive is capable of

using LBA mode commands. This reduces overhead in the driver and the

drive firmware slightly and will not impact drive operation in any other

way.

2) When Windows is configured for a temporary swap file it performs accesses

to that swap file through the PC BIOS. A potential conflicts between one

blocking factor being used for swap file accesses by the BIOS and another

blocking factor being used by all other Windows disk accesses the driver

requires special processing by the driver.  Thus the driver will not allow

the blocking factor to be changed from that selected by the BIOS when a

temporary swap file is in use by Windows.  Any attempt to change the

blocking factor in the SYSTEM.INI file will be ignored.  The driver will

continue to use block mode transfers using the blocking factor selected by

the system BIOS at power-on.

INSTALLATION INSTRUCTIONS

You must have Windows 3.1 installed on your hard disk prior to performing any

installation of the WDCDRV.386 driver. You then have two methods for

installation of the driver.  The first, and recommended, method is the

automatic installation procedure which only requires you to run the SETUP.EXE

utility from Windows 3.1. This SETUP.EXE and the associated files are located

on the distribution floppy containing the WDCDRV.386 driver. Information on

running the automated installation procedure can be found in the README.DOC

file on the WDCDRV.386 distribution floppy. The second method is manual

installation of the driver program only. The CAVIAR.EXE configuration program

and its associated help files are not installed by this procedure. You must

use the automated procedure to install these files if you wish to use them.

Manual Installation Procedure:

1. Copy WDCDRV.386 to the Windows directory of your hard disk. This

directory is normally C:\WINDOWS but may be different for your system

configuration.

2. Edit the SYSTEM.INI file as follows:

OLD:                            NEW:

[386Enh]                        [386Enh]

device=*wdctrl                  REM device=*wdctrl

device=c:\windows\wdcdrv.386

Putting a 'REM' in front of "device=*wdctrl" will cause Windows to

ignore this line. The second statement defines the path to the

Western Digital driver WDCDRV.386 which is replacing the wdctrl

FastDisk driver from Microsoft. If WDCDRV.386 is not in the c:\windows

directory then simply type in the proper path to its location in the

device statement.

3. Run Windows and select the "Control Panel" icon. Then select the

"386 Enhanced" icon. Next select the "Virtual Memory..." option.

Select "Change>>" and then check the "Use 32-bit Access" option.

The "Use 32-bit Access" option must be on in order for the

WDCDRV.386 FastDisk driver to be loaded. Windows will reboot and

WDCDRV.386 will become active.

>>>>>>  IF you do not see 32 bit Disk access flag displayed at above window

Please enter following lines in [386Enh] section of system.ini file:

[386Enh]

32BitDiskAccess=ON / Off

device=*int13

device=C:\WDCDRV.386

>>>>>>>

OPTIONAL SWITCHES:

==================

A new section called [wdcdrv] can be added in the SYSTEM.INI for optional

command line switches as follows:

[wdcdrv]

DMA=7

DMATYPE=B

In this example, the DMA channel is set to 7 with type B DMA transfers. Since

DMA channel and transfer type are automatically detected during run time,

these switches are either redundant or provide an overide to what the driver

would otherwise use.

Please note that the [wdcdrv] section must come after the [386Enh] section in

SYSTEM.INI.

[WDCDRV] OPTIONAL COMMAND LINE SWITCHES

DMA=3, 5, 6 OR 7        Default is automatically detected by the driver.

This switch allows an overide of the DMA channel

that was automatically detected by the driver.

NOTE:

Please do not use this switch if you are not sure

what DMA channel is used for the IDE connector in

your system. It is possible that there is no

DMA Channel dedicated to the IDE connector.

DMAFLAG=ON / OFF        Default is ON. Can select OFF to turn off DMA.

DMATYPE=F / B           Default is automatically determined by driver. This

switch allows an overide of the DMA type that was

automatically detected by the driver.

BLOCKINGFACTOR=4,8 or 16 This default value for this setting is automatically

determined by the driver. If the BIOS has enabled

block mode the driver will automatically use the

blocking factor programmed by the BIOS as the default.

If the BIOS did not program a blocking factor the

driver uses the value returned by the Identify Drive

command.  This switch allows an overide of the default

value chosen by the driver.

SCATTERGATHER=ON / OFF  Default is OFF. Can be enabled via this switch.

CSL=ON / OFF            Default is OFF. Required for Intel Express and

Corsair systems when DMA channel 3 is being used.

Configures DMA channel 3 properly.